Experiment 1

MPTCP =0

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Source=10.0.0.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Source= 10.0.0.2

Source=10.0.1.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 9000

r1-eth0 mtu 9000

r1-eth1 mtu 9000

Source=10.0.0.2

Source= 10.0.1.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 576

h1-eth1 mtu 576

r1-eth0 mtu 576

r1-eth1 mtu 576

Source=10.0.0.2

Source=10.0.1.2

MPTCP=0

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 9000

r1-eth0 mtu 9000

r1-eth1 mtu 9000

MPTCP =0 , MPTCP=1 with 9000 MTU each

MPTCP enabled both link  9000 MTU  vs 576 MTU

In here I used changing host interface’s MTU.

Experiment 2

Bandwidth is not set.

MPTCP=0

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

in here host 1 knows his interfaces’s MTU.  Path MTU discovery??????

Bandwidth is not set.

MPTCP=1

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Bandwidth must be set. Otherwise it does not give proper results. Even when checking different bandwidth rather than path MTU, MPTCP must work better than TCP. When removing bandwidth, it didn’t give such results. Conclusion is bandwidth must setup for even simple test.

# Day 3

Experiment 3

h1 interface 1 mtu 9000")

h1 interface 2 mtu 9000")

r1 interface 1 mtu 9000")

r1 interface 2 mtu 576")

Packets capture from router interface 3 (r1<->Server). But when investigate pcap files

When MPTCP enabled
source = 10.0.0.2   SYN=8960

Data size= 1428

Source= 10.0.1.2 MP JOIN

SYN = 8960

No packet capture records from second IP

TCP=1

MPTCP=0

Source= 10.0.0.2

SYN=8960

Source=10.0.1.2 . no records

# Day 4

MPTCP =1

R1-eth1 = 576 MTU

Rest of are default  1500

Source = 10.0.0.2

Source= 10.0.1.2

TCP

Source= 10.0.0.2

source= 10.0.1.2

TCP , there is no usage of 2nd link

Up to now in MPTCP, 2 link is not utilized. MP join opt not work. Path MTU of 1st link is 1500. Path MTU of 2nd link is 576 due to MTU of router eth1 interface.

# Day 5

## Experiment 1

MPTCP=1 MTU 9000 all

MPTCP =1 MTU 9000 vs 1500

TCP =1 MTU 9000 vs 1500

MPTCP =1 MTU 9000 vs 1500

TCP =1 MTU 9000 vs 1500

MPTCP=1 MTU 9000 all

## Experiment 2

TCP (MPTCP=0) diff MTU 9000 vs 1500

MPTCP=1 diff MTU 9000 vs 1500

MPTCP=1     9000 MTU all

MPTCP=1    1500 MTU all

# Day 6

Link 1 1500MTU

Link2 9000MTU

TCP

MPTCP

Link 1 pcap- uses exact 1500 MTU as usual

Link 2 pcap- it uses only 1500MTU instead of 9000MTU