Experiment 1

MPTCP =0

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Source=10.0.0.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Source= 10.0.0.2

Source=10.0.1.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 9000

r1-eth0 mtu 9000

r1-eth1 mtu 9000

Source=10.0.0.2

Source= 10.0.1.2

MPTCP=1

Bandwidth=100mbps

h1-eth0 mtu 576

h1-eth1 mtu 576

r1-eth0 mtu 576

r1-eth1 mtu 576

Source=10.0.0.2

Source=10.0.1.2

MPTCP=0

Bandwidth=100mbps

h1-eth0 mtu 9000

h1-eth1 mtu 9000

r1-eth0 mtu 9000

r1-eth1 mtu 9000

MPTCP =0 , MPTCP=1 with 9000 MTU each

MPTCP enabled both link  9000 MTU  vs 576 MTU

In here I used changing host interface’s MTU.

Experiment 2

Bandwidth is not set.

MPTCP=0

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

in here host 1 knows his interfaces’s MTU.  Path MTU discovery??????

Bandwidth is not set.

MPTCP=1

h1-eth0 mtu 9000

h1-eth1 mtu 576

r1-eth0 mtu 9000

r1-eth1 mtu 576

Bandwidth must be set. Otherwise it does not give proper results. Even when checking different bandwidth rather than path MTU, MPTCP must work better than TCP. When removing bandwidth, it didn’t give such results. Conclusion is bandwidth must setup for even simple test.

# Day 3

Experiment 3

h1 interface 1 mtu 9000")

h1 interface 2 mtu 9000")

r1 interface 1 mtu 9000")

r1 interface 2 mtu 576")

Packets capture from router interface 3 (r1<->Server). But when investigate pcap files

When MPTCP enabled
source = 10.0.0.2   SYN=8960

Data size= 1428

Source= 10.0.1.2 MP JOIN

SYN = 8960

No packet capture records from second IP

TCP=1

MPTCP=0

Source= 10.0.0.2

SYN=8960

Source=10.0.1.2 . no records

# Day 4

MPTCP =1

R1-eth1 = 576 MTU

Rest of are default  1500

Source = 10.0.0.2

Source= 10.0.1.2

TCP

Source= 10.0.0.2

source= 10.0.1.2

TCP , there is no usage of 2nd link

Up to now in MPTCP, 2 link is not utilized. MP join opt not work. Path MTU of 1st link is 1500. Path MTU of 2nd link is 576 due to MTU of router eth1 interface.

# Day 5

## Experiment 1

MPTCP=1 MTU 9000 all

MPTCP =1 MTU 9000 vs 1500

TCP =1 MTU 9000 vs 1500

MPTCP =1 MTU 9000 vs 1500

TCP =1 MTU 9000 vs 1500

MPTCP=1 MTU 9000 all

## Experiment 2

TCP (MPTCP=0) diff MTU 9000 vs 1500

MPTCP=1 diff MTU 9000 vs 1500

MPTCP=1     9000 MTU all

MPTCP=1    1500 MTU all

# Day 6

Link 1 1500MTU

Link2 9000MTU

TCP

MPTCP

Link 1 pcap- uses exact 1500 MTU as usual

Link 2 pcap- it uses only 1500MTU instead of 9000MTU

Day 7

Create TCP link to check whether path MTU is configured correctly

pcap file

Create bottleneck in MPTCP topology

bottelneck pacap

End host bottleneck

End host low MTU pcap

Self bottleneck low mtu pcap

self low mtu pcap

# Interim

Objectives

Make tcp connection to catch mtu detection

Create mininet script- establish single tcp link

- Change PMTUs

Which point mtu detection does

Kernal print MTUs

Which files involving to do this

-----------------------------------------------------------------------------------------------------------------------------

## Day 1

Mininet single tcp script , interfaces up as ipv6 .

Ipv6 – > addrconf.c  –>  func    addrconf_notify

Kernel log is

View log file log

View kernel msg - kernel

-----------------------------------------------------------------------------------------------------------------------------

## Day 2

Finding packets size when making IP header for data segment

net/ipv4/ip_output.c

ip_build_and_send_pkt function building IP header and sending individual IP packets,

In net/ip.h

PMTU defined as 65535U

Print skb len in kernel to check what is there. net/ipv4/ip_output.c -- ip_build_and_send_pkt()

Result

It print as 52

But data is not transferred

When mtu size change to 1500 for all

It transferred data.

Change MTU to 9000 for all – data transferred

Eventhough interface MTU size 1500 or 9000 skb->len give 52     ?

When create bottleneck data is not transferred

internet note:- tcp_mtu_probing = 2 (Aggressive Probing)

- In addition to responding to "Fragmentation Needed" messages, the kernel periodically sends larger packets to test whether a larger MTU is now supported on the path.

- This "aggressive probing" approach allows the kernel to adjust the MTU up or down, depending on network path changes, which is especially useful for dynamic network environments.

- If a larger MTU becomes possible due to network changes, it improves TCP performance by reducing the number of packets needed to transfer data.

-----------------------------------------------------------------------------------------------------------------------------

## Day 3

Finding where does PATH MTU value decision takes place for stablished tcp link

Enabled PMTUD = sysctl net.ipv4.ip_no_pmtu_disc= 0

Enabled tcp mtu probe =  sysctl net.ipv4.tcp_mtu_probing= 1

Need to find where does ICMP handles PMTU values

Ipv4/icmp.c

internet note:- kernel handles ICMP messages in the icmp.c file, where it inspects the type of ICMP error. Specifically, the PMTU-related ICMP error (Type 3, Code 4) indicates that a packet is too large for the next hop’s MTU.

Ipv4/icmp.h/icmp_unreach()

Modified this

passing the ICMP message along with the MTU info (info) to the transport layer, which adjusts the MTU for future packets.

## Day 4

Mininet script python file

Finally got some results on kernel print

Results

Changes:-

Iperf run on 9000 mtu and verbose mode (-d)

MTU configuaration

Backend configurations

Iperf output